### Step 1: Create a VPC

- Create a VPC with the CIDR block 59.168.0.0/16.

### Step 2: Create Subnets

Create two subnets within the VPC:

- Public Subnet: 59.168.0.0/24

- Private Subnet: 59.168.1.0/24

### Step 3: Create and Attach an Internet Gateway (IGW)

- Create an Internet Gateway (IGW).

- Attach the IGW to the VPC.

### Step 4: Create Route Tables

Public Route Table

- 59.168.0.0/16 → Local (VPC)

- 0.0.0.0/0 → Internet Gateway (IGW)

Associate this route table with the Public Subnet.

Private Route Table

- 59.168.0.0/16 → Local (VPC)

Associate this route table with the Private Subnet.

### Step 5: Launch an EC2 Instance

- Launch an EC2 instance in the Public Subnet.

- Enable Auto-assign Public IPv4 Address.

- Attach the appropriate security group (allow SSH from your IP).

### Step 6: Connect to the Instance

- Connect to the EC2 instance using SSH via its public IP address.

- Verify internet connectivity by pinging Google's DNS server:

ping 8.8.8.8

ping is successful, the instance has outbound internet access through the Internet Gateway.